Christopher King

2018141521058

Mechanical Design 1

Class Section 01

11/18/2020

# Problem 1

Solution:

For this question, we are asked to calculate the allowable load P using the Coulomb-Mohr theory and a factor of safety of n=2.5.

Since the load acts in a plane of symmetry, there are three independent equations of equilibrium.

The internal forces at the section are found using the equations of equilibrium as follows:

+→Fx=0⇒V=0

+↑Fy=0⇒F-P=0⇒F=P

+↺MA-B=0⇒M-0.4+0.11P=0⇒M=0.51P

Thus, there are two internal forces on the section, an axial force P, which produces a constant normal stress a σ1=FA over the section, and a bending moment M, which produces a linear variation of normal stress σ2=-MxI over the section. The cross-sectional area A and the second moment I of the cross-sectional area with respect to the centroidal axis NA are

A=80 mm×180 mm+240 mm×120 mm=0.0432 m2

I=112×240 mm×120 mm3+110 mm-60 mm2×240 mm×120 mm+112×80 mm×180 mm3+90 mm+10 mm2×80 mm×180 mm=2.8944×10-4 m4

The stresses σ1 and σ2 due to the internal forces are

σ1=FA=P0.0432 m2

The maximum tensile flexural stress occurs at the left edge of section NA (point A) and is

σ2A=-MxAI=-0.51P×-110 mm2.8944×10-4 m4

The maximum compressive flexural stress occurs at the right edge of section NA (point B) and is

σ2B=-MxBI=-0.51P×190 mm2.8944×10-4 m4

The distributions of stresses a 1 and a2 are shown in figures above, respectively. Superimposing the normal stresses at points A and B gives

σA=σ1+σ2A=P0.0432 m2+0.51P×110 mm2.8944×10-4 m4=216.971 P (T)

σB=σ1+σ2B=P0.0432 m2-0.51P×190 mm2.8944×10-4 m4=-311.64 P=311.64 P (C)

⇒σA≤SutnσB≤-Sucn⇒216.971 P≤170 MPa2.5-311.64 P≤650 MPa2.5⇒P≤313.4 kNP≤834.3 kN

Therefore, the allowable load P using the Coulomb-Mohr theory and a factor of safety of n=2.5 is equal to 313.4 kN.

# Problem 2

Solution:

For this question, we are asked to determine the maximum possible load that can be applied before the plate (a) yields, and (b) has uncontrollable crack growth.

F=SynA=950 MPa1×12 mm×100 mm-16 mm=957.6 kN

ab=16 mm100 mm=0.16hb=100 mm100 mm=1⟹β=1.3

K=KIc=βσπa=βFπaA

F=KIcAβπa=80 MPam×12 mm×100 mm1.3×π×16 mm=329.38 kN