Christopher King

2018141521058

Mechanical Design 2

Class Section 01

09/23/2021

# Problem 1

A solid square rod is cantilevered at one end. The rod is 0.6 m long and supports a completely reversing transverse load at the other end of ±2 kN. The material is AISI 1080 hot-rolled steel. If the rod must support this load for 10,000 cycles with a design factor of ~1.5, what dimension should the square cross section have? Since the size is not yet known, assume a typical value of kb = 0.85 and verify its correctness later. Neglect any stress concentrations at the support end.

Solution:

For this question, we are asked to determine the dimension that the square cross section should have.

From Table A-20, I can get that the ultimate strength of AISI 1080 hot-rolled steel is equal to

Sut=770 MPa=112 ksi

Therefore, the endurance limit is equal to

Se'=0.5Sut=0.5×770 MPa=385 MPa

Next, we consider to modify the endurance limit.

Surface Condition (hot-rolled):

ka=aSutb=57.7×770-0.718=0.4883

Size Effect:

kb=0.85

Loading Effect (bending):

kc=1

Temperature Effect (room temperature):

kd=1

Reliability Effect:

ke=1

Therefore, the modified endurance limit is equal to

Se=kakbkckdkeSe'=0.4883×0.85×1×1×1×385 MPa=159.79 MPa

From Figure 6-18, I can know that the fatigue strength fraction is equal to

f=0.83

Therefore,

a=fSut2Se=0.83×770 MPa2159.79 MPa=2.5561×103 MPa

b=-13logfSutSe=-13×log0.83×770 MPa159.79 MPa=-0.2007

Sf=aNb=2.5561×103 MPa×10000-0.2007=402.6221 MPa

From the force diagram, I can know that

Mmax=2 kN×0.6 m=1200 N∙m

σa=σmax=McI=Mb2bb312=6Mb3=7200b3 Pa

And

Sf=nσa

402.6221 MPa=1.5×7200b3 Pa

b=0.0299 m

Check for the size effect:

de=0.808hb=0.808b2=0.808b=0.0242 m=24.2 mm

kb=d7.62-0.107=24.27.62-0.107=0.8837

The result doesn’t match our guess.

Therefore, I conduct the iteration to determine the dimension of the beam.

clc; clear all;

kb = 0.85;

for i = 1:20

Sut = 770;

f = 0.83;

ka = 57.7*770^(-0.718);

Se = ka*kb*385;

a=(f*Sut)^2/Se;

b=-1/3*log10((f*Sut)/Se);

Sf = a*10000^b;

bb = (1.5*7200/(Sf*1E6))^(1/3);

de=0.808*bb*1000;

kb = (de/7.62)^(-0.107);

end

After conducting 20 iterations, the value of kb is converged to 0.8842 and the value of size b is converged to 0.0298 m=29.8 mm.

# Problem 2

The shaft shown in the figure is machined from AISI 1040 CD steel. The shaft rotates at 1600 rpm and is supported in rolling bearings at A and B. The applied forces are F1 = 2500 lbf and F2 = 1000 lbf. Radius of all fillets is 1/16 in R.

Determine the minimum fatigue factor of safety based on achieving infinite life. If infinite life is not predicted, estimate the number of cycles to failure. Also check for yielding.

Solution:

For this question, we are asked to determine the minimum fatigue factor of safety based on achieving infinite life. If infinite life is not predicted, estimate the number of cycles to failure. Also check for yielding.

From Table A-20, I can get that the ultimate strength and yield strength of AISI 1040 CD steel is equal to

Sut=590 MPa=85 ksiSy=490 MPa=71 ksi

Therefore, the endurance limit is equal to

Se'=0.5Sut=0.5×85 ksi=42.5 ksi

Next, we consider to modify the endurance limit.

Surface Condition (cold drawn):

ka=aSutb=2.70×85-0.265=0.8319

Size Effect (d=158 in):

kb=d0.3-0.107=1580.3-0.107=0.8346

Loading Effect (bending):

kc=1

Temperature Effect (room temperature):

kd=1

Reliability Effect:

ke=1

Therefore, the modified endurance limit is equal to

Se=kakbkckdkeSe'=0.8319×0.8346×1×1×1×42.5 ksi=29.5085 ksi

From the force diagram, I can know that the bending moment at the second fillet is equal to

M=2000 lbf×10.5 in-2500 lbf×2.5 in=14750 lbf∙in

σ=McI=14750 lbf∙in×158 in2π158 in464=35.0132 ksi

According to Figure A-15-9, when rd=116 in158 in=0.03846 and Dd=178 in158 in=1.15, the stress concentration factor is equal to

Kt=1.95

a=0.246-3.08×10-3Sut+1.51×10-5Sut2-2.67×10-8Sut3=0.246-3.08×10-3×85+1.51×10-5×852-2.67×10-8×853=0.0769

⇒a=0.0059

The fatigue stress concentration is equal to

Kf=1+Kt-11+ar=1+1.95-11+0.0059116=1.7265

Therefore, the fatigue factor of safety is equal to

nf=SeKfσ=29.5085 ksi1.7265×35.0132 ksi=0.4881

Hence, I can know that infinite life is not predicted.

From Figure 6-18, I can know that the fatigue strength fraction is equal to

f=0.867

Therefore,

a=fSut2Se=0.867×85 ksi229.5085 ksi=184.0470 ksi

b=-13logfSutSe=-13×log0.867×85 ksi29.5085 ksi=-0.1325

Sf=aNb

Kfσrev=aNb

1.7265×35.0132 ksi=184.0470 ksi×N-0.1325

⇒N=4459.9

The safety factor for yield strength is equal to

ny=SyKfσ=71 ksi1.7265×35.0132 ksi=1.1745

# Problem 3

Shaft A, made of AISI 1020 hot-rolled steel, is welded to a fixed support and is subjected to loading by equal and opposite forces F via shaft B. A theoretical stress concentration factor Kts of 1.6 is induced in the shaft by the 1/8-in weld fillet. The length of shaft A from the fixed support to the connection at shaft B is 2 ft. The load F cycles from 150 to 500 lbf.

- For shaft A, find the factor of safety for infinite life using the modified Goodman fatigue failure criterion.

- Repeat part (a) using the Gerber fatigue failure criterion.

Solution:

- For this question, we are asked to find the factor of safety for infinite life using the modified Goodman fatigue failure criterion for shaft A.

From Table A-20, I can get that the ultimate strength of AISI 1020 hot-rolled steel is equal to

Sut=380 MPa=55 ksiSy=210 MPa=30 ksi

Therefore, the endurance limit is equal to

Se'=0.5Sut=0.5×55 ksi=27.5 ksi

Next, we consider to modify the endurance limit.

Surface Condition (hot-rolled):

ka=aSutb=14.4×55-0.718=0.8106

Size Effect (d=78 in):

de=0.370d=0.370×78 in=0.3237 in

kb=d0.3-0.107=0.32370.3-0.107=0.9919

Loading Effect (torsion):

kc=0.59

Temperature Effect (room temperature):

kd=1

Reliability Effect:

ke=1

Therefore, the modified endurance limit is equal to

Sse=kakbkckdkeSe'=0.8106×0.9919×0.59×1×1×27.5 ksi=13.0445 ksi

And because the stress in this question is torsion,

a=0.190-2.51×10-3Sut+1.35×10-5Sut2-2.67×10-8Sut3=0.246-3.08×10-3×55+1.51×10-5×552-2.67×10-8×553=0.0883

⇒a=0.0078

The fatigue stress concentration is equal to

Kf=1+Kt-11+ar=1+1.6-11+0.0078116=1.4800

From the question, I can know that

Fmax=500 lbfFmin=150 lbf⇒Tmax=1000 lbf∙inTmin=300 lbf∙in

⇒τmax=KfTmaxcJ=Kf16Tmaxπd3=1.4800×16×1000 lbf∙inπ×78 in3=11.2518 ksiτmin=KfTmincJ=Kf16Tminπd3=1.4800×16×300 lbf∙inπ×78 in3=3.3755 ksi

⇒τm=τmax+τmin2=11.2518 ksi+3.3755 ksi2=7.3137 ksiτa=τmax-τmin2=11.2518 ksi-3.3755 ksi2=3.9381 ksi

The safety factor for yield strength is equal to

ny=Sy2τmax=30 ksi211.2518 ksi=1.3331

And, in constructing the Goodman diagram, I use

Ssu=0.67Sut=0.67×55 ksi=36.8500 ksi

Then implement the Modified Goodman failure criterion:

nf=1τaSse+τmSsu=13.9381 ksi13.0445 ksi+7.3137 ksi36.8500 ksi=1.9985

- For this question, we are asked to find the factor of safety for infinite life using the Gerber fatigue failure criterion for shaft A.

Implement the Gerber fatigue failure criterion:

nf=12Ssuτm2τaSse-1+1+2τmSseSsuτa2=12×36.8500 ksi7.3137 ksi2×3.9381 ksi13.0445 ksi×-1+1+2×7.3137 ksi×13.0445 ksi36.8500 ksi×3.9381 ksi2=2.4981