Christopher King

2018141521058

Mechanical Design 2

Class Section 01

09/26/2021

# Problem 1

The job is to conduct a first-cut shaft diameter estimation. Designed shaft needs to transmit 1000 N-m torque with superimposed 250 N-m alternating torque due to torsional vibration. Shaft material is a heat- treated alloy steel with Sut=1.2GPa and Sy=1.0GPa. The shaft has a shoulder with designated D/d=1.2 and r/d=0.05. Shaft surface demands a good quality ground finish. Reliability target of the designed shaft is 95 percent.

- What is the minimal diameter required for infinite life?

- Identify your assumptions made to get estimated diameter.

Solution:

- For this question, we are asked to determine the minimal diameter required for infinite life.

Mm=0 N⋅m

Ma=0 N⋅m

Tm=1000 N⋅m

Ta=250 N⋅m

Assume 2.79≤d≤51 mm

And

Sut=1.2 GPa=174.0452927 ksiSy=1.0 GPa=145.0377439 ksi

Therefore, the endurance limit is equal to

Se'=0.5Sut=0.5×1.2 GPa=600 MPa

Next, we consider to modify the endurance limit.

Surface Condition (ground):

ka=aSutb=1.58×1200-0.085=0.8648

Size Effect:

kb=d×10007.62-0.107

Loading Effect (torsion):

kc=0.59

Temperature Effect (room temperature):

kd=1

Reliability Effect (95%):

ke=0.868

Therefore, the modified endurance limit is equal to

Sse=kakbkckdkeSe'=0.8648×d×10007.62-0.107×0.59×1×0.868×600 MPa

From Table A-15-8, I can know that the stress concentration factor for D/d=1.2 and r/d=0.05 round shaft with shoulder fillet is equal to

Kts=1.6

And because the stress in this question is torsion,

a=0.190-2.51×10-3Sut+1.35×10-5Sut2-2.67×10-8Sut3=0.190-2.51×10-3×174.0452927+1.35×10-5×174.04529272-2.67×10-8×174.04529273=0.0213

⇒a=4.5456×10-4

The fatigue stress concentration is equal to

Kfs=1+Kts-11+ar=1+1.6-11+4.5456×10-40.05d×100×0.39370078740

And

1n=σa'Se+σm'Sut

For the minimal diameter required for infinite life,

n=1

Therefore,

1=σa'Se+σm'Sut

1=32KfMaπd32+316KfsTaπd3212Se+32KfMmπd32+316KfsTmπd3212Sut

1=32Kf⋅0πd32+316KfsTaπd3212Se+32Kf⋅0πd32+316KfsTmπd3212Sut

1=316KfsTaπd3212Se+316KfsTmπd3212Sut

1=163Kfsπd3TaSe+TmSut

1=1631+1.6-11+4.5456×10-40.05d×100×0.39370078740πd3×250 N⋅m0.8648×d×10007.62-0.107×0.59×1×0.868×600 MPa+1000 N⋅m1.2 GPa

⇒d=0.02974 m=29.74 mm

- For this question, we are asked to identify your assumptions made to get estimated diameter.

Because 2.79≤d=29.74≤51 mm, the assumption is satisfied.

# Problem 2

The rotating solid steel shaft is simply supported by bearings at points B and C and is driven by a gear (not shown) which meshes with the spur gear at D, which has a 150-mm pitch diameter. The force F from the drive gear acts at a pressure angle of 20°. The shaft transmits a torque to point A of TA = 340 N · m. The shaft is machined from steel with Sy = 420 MPa and Sut = 560 MPa.

Using a factor of safety of 2.5, determine the minimum allowable diameter of the 250-mm section of the shaft based on (a) a static yield analysis using the distortion energy theory and (b) a fatigue-failure analysis. Assume sharp fillet radii at the bearing shoulders for estimating stress-concentration factors.

Solution:

For this question, we are asked to determine the minimum allowable diameter of the 250-mm section of the shaft based on (a) a static yield analysis using the distortion energy theory and (b) a fatigue-failure analysis. Assume sharp fillet radii at the bearing shoulders for estimating stress-concentration factors.

From the force analysis, I can know that

Fcos20°R=TA

⟹F=TARcos20°=340 N⋅m150 mm2cos20°=4.8243×103 N

And the maximum bending moment occurs at point C, which is equal to

MC=FxCD=4.8243×103 N×100 mm=4.8243×102 N⋅m

Therefore, we can know that at point C:

Mm=0 N⋅m

Ma=4.8243×102 N⋅m

Tm=340 N⋅m

Ta=0 N⋅m

For sharp fillet, Kt=2.7 and Kts=2.2.

And,

a=0.246-3.08×10-3Sut+1.51×10-5Sut2-2.67×10-8Sut3=0.246-3.08×10-3×81.22+1.51×10-5×81.222-2.67×10-8×81.223=0.081146

⇒a=6.58462×10-3

The fatigue stress concentration is equal to

Kf=1+Kt-11+ar=1+2.7-11+6.58462×10-30.02d×100×0.39370078740

a=0.190-2.51×10-3Sut+1.35×10-5Sut2-2.67×10-8Sut3=0.190-2.51×10-3×81.22+1.35×10-5×81.222-2.67×10-8×81.223=0.06089

⇒a=3.7072×10-3

The fatigue stress concentration is equal to

Kfs=1+Kts-11+ar=1+2.2-11+3.7072×10-30.02d×100×0.39370078740

Assume q=0.8 and qs=0.9.

So,

Kf=1+qKt-1=1+0.82.7-1=2.4

Kfs=1+qsKts-1=1+0.92.2-1=2.1

σmax'=σm+σa2+3τm+τa212=32KfMm+Maπd32+316KfsTm+Taπd3212=32×1+2.7-11+6.58462×10-30.02d×100×0.39370078740×4.8243×102 N⋅mπd32+316×1+2.2-11+3.7072×10-30.02d×100×0.39370078740×340 N⋅mπd3212

And

nσmax'=Sy

2.5×32×1+2.7-11+6.58462×10-30.02d×100×0.39370078740×4.8243×102 N⋅mπd32+316×1+2.2-11+3.7072×10-30.02d×100×0.39370078740×340 N⋅mπd3212=420 MPa

⇒d=41.61 mm

Assume d≥51 mm

And

Sut=560 MPaSy=420 MPa

Therefore, the endurance limit is equal to

Se'=0.5Sut=0.5×560 MPa=280 MPa

Next, we consider to modify the endurance limit.

Surface Condition (machined):

ka=aSutb=4.51×560-0.265=0.843

Size Effect:

kb=1.51d×1000-0.157

Loading Effect (bending):

kc=1

Temperature Effect (room temperature):

kd=1

Reliability Effect:

ke=1

Therefore, the modified endurance limit is equal to

Sse=kakbkckdkeSe'=0.843×1.51d×1000-0.157×1×1×1×280 MPa

Hence,

d=16nπ1Se4KfMa2+3KfsTa212+1Sut4KfMm2+3KfsTm21213=16×2.5π10.843×1.51d×1000-0.157×1×1×1×280 MPa41+2.7-11+6.58462×10-30.02d×100×0.39370078740×4.8243×102 N⋅m2+31+2.2-11+3.7072×10-30.02d×100×0.39370078740×0212+1560 MPa41+2.7-11+6.58462×10-30.02d×100×0.39370078740×02+31+2.2-11+3.7072×10-30.02d×100×0.39370078740×340 N⋅m21213

⇒d=55.37 mm

# Problem 3

The torque to be transmitted through the key from the gear to the shaft is T = 2819 in-lbf. The nominal shaft diameter supporting the gear is 1.00 in. Specify a square key for torque transmission, using a factor of safety of 1.1. Use 1020 CD steel for the key material and DET theory as the failure criteria for safety factor calculation.

Solution:

For this question, we are asked to specify a square key for torque transmission, using a factor of safety of 1.1.

From Table 7–6, a 14-in square key is selected. Choose 1020 CD steel for the key material, with a yield strength of 57 kpsi.

From Fig. 7–19, the force F at the surface of the shaft is

F=Tr=2819 in⋅lbf1 in2=5638 lbf

By the distortion-energy theory, the shear strength is

Ssy=0.577Sy=0.577×57 kpsi=32.889 ksi

Failure by shear across the area ab will create a stress of τ=F/tl. Substituting the strength divided by the design factor for τ gives

Ssyn=Ftl

32.889 ksi1.1=5638 lbf0.25l

l=0.754 in

To resist crushing, the area of one-half the face of the key is used:

Syn=Ftl/2

57 ksi1.1=5638 lbf0.25l/2

l=0.87 in

Failure by crushing the key is the dominant failure mode, so it defines the necessary length of the key to be l=0.87 in.