Christopher King

2018141521058

Mechanical Design 2

Class Section 01

10/25/2021

# Problem 1

(Continued from HW06, Question 02) 1. An angular-contact, inner ring rotating, 02-series ball bearing is required for an application in which the life requirement is 40 kh at 520 rev/min. The design radial load is 725 lbf. The application factor is 1.4. The reliability goal is 0.95. Determine

- multiple of rating life xD,

- minimum C10, catalog rating C10, and bearing selection corresponding to Table 11–2; and

- the existing reliability in service.

Solution:

xD=LDLR=60LDnDL10=40 kh×520 rpm×60106=1248

Minimum C10

C10=afFDxDx0+θ-x01-RD1b1a=1.4×725 lbf×12480.02+4.4391-0.950.67413=12.890 klbf=57.3376 kN

I select the bear with 65 mm bore in Table 11-2 with catalog rating C10=63.7 kN.

R=exp-xDafFDC10a-x0θ-x0b=exp-12481.4×725 lbf63.7 kN3-0.024.4391.483=0.9697

# Problem 2

(Continued from HW06, Question 03) The other bearing on the shaft of Question 02 to be a 03-series cylindrical roller bearing with inner ring rotating. For a 2235-lbf radial load, find the catalog rating C10 with which to enter Table 11–3. The reliability goal is 0.90. Choose a bearing and estimate its reliability in use.

Solution:

C10=afFDxDx0+θ-x01-RD1b1a=1.4×2235 lbf×12480.02+4.4391-0.900.6741103=26.887 klbf=119.6 kN

I select the bear with 60 mm bore in Table 11-3 with catalog rating C10=123 kN.

R=exp-xDafFDC10a-x0θ-x0b=exp-12481.4×2235 lbf123 kN103-0.024.4391.483=0.9168

# Problem 3

Questions 01 and 02 raise the question of the reliability of the bearing pair on the shaft. What is the reliability of the two bearings (probability that either or both will not fail) as a result of your decisions in Probs. 1 and 2?

Solution:

R=R1R2=0.9697×0.9168=0.8890

# Problem 4

The worm shaft shown in the figure transmits 1.2 hp at 500 rev/min. A static force analysis gave the results shown in the figure. Bearing A is to be an angular-contact ball bearing selected from Table 11–2, mounted to take the 555-lbf thrust load. The bearing at B is to take only the radial load, so an 02-series cylindrical roller bearing from Table 11–3 will be employed. Use an application factor of 1.2, a desired life of 30 kh, and a combined reliability goal of 0.99, assuming distribution data from manufacturer 2 in Table 11–6.

Specify each bearing.

Solution:

For bear B:

xD=LDLR=60LDnDL10=30 kh×500 rpm×60106=900

Fr=362+672=338.3281 N

Try R=1,

C10=afFDxDx0+θ-x01-RD1b1a=1.2×338.3281 N×9000.02+4.4391-10.6741103=10.104 kN

The smallest C10 in Table 11-3 is 16.8 kN. Therefore, I select the bear with 25 mm bore in Table 11-3.

For bear A:

Fr=362+672=956.5228 N

Fa=2468.8 N

Try 85 mm bore in Table 11-2 with catalog rating C10=90.4 kN.

FaC0=2468.8 N63.0 kN=0.0392

Interpolate using Table 11-1:

X2=0.56

Y2=1.8781

Therefore,

Fe=0.56×956.5228 N+1.8781×2468.8 N=5172.3 N

C10=afFexDx0+θ-x01-RD1b1a=1.2×5172.3 N×9000.02+4.4391-0.990.67413=99.388 kN>90.4 kN

Therefore, we try 90 mm bore in Table 11-2 with catalog rating C10=106 kN again.

FaC0=2468.8 N73.5 kN=0.0336

Interpolate using Table 11-1:

X2=0.56

Y2=1.9341

Therefore,

Fe=0.56×956.5228 N+1.9341×2468.8 N=5310.5 N

C10=afFexDx0+θ-x01-RD1b1a=1.2×5310.5 N×9000.02+4.4391-0.990.67413=102.04 kN<106 kN

Therefore, 90 mm bore in Table 11-2 is suitable.