Christopher King

2018141521058

Mechanical Design 2

Class Section 01

11/05/2021

# Problem 1

A full journal bearing has a shaft journal diameter of 25 mm with a unilateral tolerance of -0.01 mm. The bushing bore has a diameter of 25.04 mm with a unilateral tolerance of 0.03 mm. The l⁄d ratio is unity. The bushing load is 1.25 kN, and the journal rotates at 1200 rev/min. Analyze the minimum clearance assembly if the average viscosity is 50 mPa-s to find the minimum oil film thickness, the power loss, and the percentage of side flow.

Solution:

Known:

dmax=25 mm=0.025 m

dmin=24.99 mm

Dmin=25.04 mm=0.02504 m

Dmax=25.07 mm

W=1.25 kN=1250 N

N=1200 rev/min=20 rev/s

μ=50 mPa⋅s=0.05 Pa⋅s

ld=1

Therefore,

2cmin=Dmin-dmax=0.04 mm

cmin=0.02 mm=2×10-5 m

l=d=25 mm=0.025 m

r=d2=25 mm2=0.0125 m

Shaft average pressure:

P=W2rl=2.0000×106 Pa

Sommerfeld Number:

S=rc2μNP=0.1953

Figure 12-16: minimum film thickness variable

h0c=0.525

Therefore, minimum film thickness:

h0=0.525c=0.525×0.02 mm=0.0105 mm

Figure 12-18: coefficient of friction variable

rcf=4.5

Therefore, coefficient of friction:

f=4.5cr=4.5×2×10-5 m0.0125 m=0.0072

Journal friction torque:

T=fWr=0.0072×1250 N×0.0125 m=0.1125 N⋅m

Power loss:

ploss=2πNT=2π×20 rev/s×0.1125 N⋅m=14.1372 W

Figure 12-20: flow ratio (the percentage of side flow)

QsQ=0.58=58%

# Problem 2

A sleeve bearing supports a load of 700 lbf and has a journal speed of 3600 rev/ min. Bearing length is 1.25-in and diameter is 1.25-in as well. An SAE 10 oil is used having an average temperature of 160°F. Using Raimondi–Boyd charts (Fig. 12-16 as shown below), estimate the radial clearance for minimum coefficient of friction f and for maximum load-carrying capacity W. The difference between these two clearances is called the clearance range. Define the range per your clearance.

Solution:

Prof. Sui told us we don’t need to do this question.

# Problem 3

A full journal bearing has a shaft diameter of 3.000 in with a unilateral tolerance of -0.0004 in. The bushing has a bore diameter of 3.003 in with a unilateral tolerance of 0.0012 in.

Lubricant used in bearing is the SAE 40 oil, which is supplied to the bearings at pump pressure of 60 psig.

The l/d ratio of the full bearing is unity. However, the bearing has a central axial-groove and its width is 20 percent of the full bearing width.

The radial bearing load is 1200 lbf while the shaft is rotating at 1800 rpm.

- What is the full range of the diametric clearance? (10 points)

- Calculate (1) the average film temperature, (2) the peak film pressure, (3) the minimum film thickness, (4) the heat loss rate, and (5) the lubricant side-flow rate under the following clearance conditions:

    - minimum clearance, and

    - maximum clearance.

Iterate till temperature between two iterations are within 5 degF. Tabulate the calculated parameters versus corresponding clearance. Compare characteristics of your data to the trends in Figure 12-25 and outline your observations. (20 points)

- What is the required capacity of the oil pump to prevent the bearing from starvation within the designed bearing clearance range? (10 points)

- It is intended to maintain the sump temperature at 150degF during operation and an oil cooler is used to remove the heat from returning oil. Specify the required capacity of the oil cooler within the designed bearing clearance range. (10 points)

- Use Trumpler’s design criteria to evaluate the minimum film thickness and oil temperature rise of the designed journal bearing. Judge whether the bearing will survive the operation or not. (10 points)

Solution:

Known:

dmax=3.000 in

dmin=2.9996 in

Dmin=3.003 in

Dmax=3.0042 in

Lubricant: SAE 40 oil.

ps=60 psig

ld=1

However, the bearing has a central axial-groove and its width is 20 percent of the full bearing width.

W=1200 lbf

N=1800 rpm=30 rev/s

- Therefore,

2cmin=Dmin-dmax=0.003 in

2cmax=Dmax-dmin=0.0046 in

The full range of the diametric clearance:

2c∈0.003 in, 0.0046 in

l'd=1-20%2=0.4

l'=0.4d=0.4×3.000 in=1.200 in

r=d2=3.000 in2=1.500 in

Shaft average pressure:

P=W/22rl'=W4rl'=166.6667 psi

[minimum clearance]

Initial guess: average film temperature TF=170℉.

SAE 40 oil viscosity:

μ=μ0expbTF+95=0.0121×10-6 reyn×exp1474.4170+95=3.1557×10-6 reyn

Sommerfeld Number:

S=rc2μNP=0.5680

Figure 12-16: eccentricity ratio

ϵ=0.54

Figure 12-18: coefficient of friction variable

rcf=13

∆TF=0.0123frcSW21+1.5ϵ2psr4=299.5650℉

Estimated side flow oil temperature:

Tav=Ts+∆TF2=150℉+299.5650℉2=299.7825℉

2nd Iteration: revised film temperature TF=300℉.

SAE 40 oil viscosity:

μ=μ0expbTF+95=0.0121×10-6 reyn×exp1474.4299.7825+95=0.5057×10-6 reyn

Sommerfeld Number:

S=rc2μNP=0.0910

Figure 12-16: eccentricity ratio

ϵ=0.84

Figure 12-18: coefficient of friction variable

rcf=3.6

∆TF=0.0123frcSW21+1.5ϵ2psr4=9.2823℉

Estimated side flow oil temperature:

Tav=Ts+∆TF2=150℉+9.2823℉2=154.6411℉

3rd Iteration: revised film temperature TF=155℉.

SAE 40 oil viscosity:

μ=μ0expbTF+95=4.4064×10-6 reyn

Sommerfeld Number:

S=rc2μNP=0.7931

It is divergent. Hence, initial guess: average film temperature TF=250℉.

SAE 40 oil viscosity:

μ=μ0expbTF+95=0.0121×10-6 reyn×exp1474.4250+95=0.86855×10-6 reyn

Sommerfeld Number:

S=rc2μNP=0.1563

Figure 12-16: eccentricity ratio

ϵ=0.76

Figure 12-18: coefficient of friction variable

rcf=3.3

∆TF=0.0123frcSW21+1.5ϵ2psr4=16.1187℉

Estimated side flow oil temperature:

Tav=Ts+∆TF2=150℉+16.1187℉2=158.0593℉

Initial guess: average film temperature TF=200℉.

SAE 40 oil viscosity:

μ=μ0expbTF+95=0.0121×10-6 reyn×exp1474.4200+95=1.7922×10-6 reyn

Sommerfeld Number:

S=rc2μNP=0.3226

Figure 12-16: eccentricity ratio

ϵ=0.66

Figure 12-18: coefficient of friction variable

rcf=8

∆TF=0.0123frcSW21+1.5ϵ2psr4=91.0144℉

Estimated side flow oil temperature:

Tav=Ts+∆TF2=150℉+91.0144℉2=195.5072℉

2nd Iteration: revised film temperature TF=197℉.

SAE 40 oil viscosity:

μ=μ0expbTF+95=0.0121×10-6 reyn×exp1474.4197+95=1.8866×10-6 reyn

Sommerfeld Number:

S=rc2μNP=0.3396

Figure 12-16: eccentricity ratio

ϵ=0.64

Figure 12-18: coefficient of friction variable

rcf=8

∆TF=0.0123frcSW21+1.5ϵ2psr4=98.1246℉

Estimated side flow oil temperature:

Tav=Ts+∆TF2=150℉+98.1246℉2=199.0623℉

3rd Iteration: revised film temperature TF=198℉.

SAE 40 oil viscosity:

μ=μ0expbTF+95=0.0121×10-6 reyn×exp1474.4198+95=1.8543×10-6 reyn

Sommerfeld Number:

S=rc2μNP=0.3338

Figure 12-16: eccentricity ratio

ϵ=0.64

Figure 12-18: coefficient of friction variable

rcf=8

∆TF=0.0123frcSW21+1.5ϵ2psr4=96.4480℉

Estimated side flow oil temperature:

Tav=Ts+∆TF2=150℉+96.4480℉2=198.2240℉

Therefore, the average film temperature is equal to TF=198℉.

Iteration Table:

| Trial Tf | μ (10-6) | S | frc | ϵ | ∆Tf | Tav |
| --- | --- | --- | --- | --- | --- | --- |
| 200 | 1.7922 | 0.3226 | 8 | 0.66 | 91.0144 | 195.5072 |
| 197 | 1.6222 | 0.3396 | 8 | 0.64 | 98.1246 | 199.0623 |
| 198 | 1.5961 | 0.3338 | 8 | 0.64 | 96.4480 | 198.2240 |

Figure 12-21: maximum-film-pressure ratio

ppmax=0.32

pmax=520.8333 psi

The peak film pressure:

ptotal=pmax+ps=520.8333 psi+60 psi=580.8333 psi

The minimum film thickness:

h0=1-ϵc=5.4×10-4 in

The lubricant side-flow rate:

Qs=πpsrc33μl'1+1.5ϵ2=0.2308 in3/s

The heat loss rate:

Hloss=ρCpQs∆T=0.0311 lbm/in3×0.42 Btu/lbm⋅℉×0.2308 in3/s×96.4480℉=0.2907 Btu/s=1046.6 Btu/h

[maximum clearance]

Initial guess: average film temperature TF=200℉.

SAE 40 oil viscosity:

μ=μ0expbTF+95=0.0121×10-6 reyn×exp1474.4200+95=1.7922×10-6 reyn

Sommerfeld Number:

S=rc2μNP=0.1372

Figure 12-16: eccentricity ratio

ϵ=0.80

Figure 12-18: coefficient of friction variable

rcf=4.5

∆TF=0.0123frcSW21+1.5ϵ2psr4=18.3688℉

Estimated side flow oil temperature:

Tav=Ts+∆TF2=150℉+18.3688℉2=159.1844℉

2nd Iteration: revised film temperature TF=180℉.

SAE 40 oil viscosity:

μ=μ0expbTF+95=0.0121×10-6 reyn×exp1474.4180+95=2.5777×10-6 reyn

Sommerfeld Number:

S=rc2μNP=0.1973

Figure 12-16: eccentricity ratio

ϵ=0.77

Figure 12-18: coefficient of friction variable

rcf=6

∆TF=0.0123frcSW21+1.5ϵ2psr4=36.5447℉

Estimated side flow oil temperature:

Tav=Ts+∆TF2=150℉+36.5447℉2=168.2724℉

3rd Iteration: revised film temperature TF=173℉.

SAE 40 oil viscosity:

μ=μ0expbTF+95=0.0121×10-6 reyn×exp1474.4173+95=2.9652×10-6 reyn

Sommerfeld Number:

S=rc2μNP=0.2270

Figure 12-16: eccentricity ratio

ϵ=0.74

Figure 12-18: coefficient of friction variable

rcf=6.5

∆TF=0.0123frcSW21+1.5ϵ2psr4=47.2402℉

Estimated side flow oil temperature:

Tav=Ts+∆TF2=150℉+47.2402℉2=173.6201℉

Therefore, the average film temperature is equal to TF=173℉.

Iteration Table:

| Trial Tf | μ (10-6) | S | frc | ϵ | ∆Tf | Tav |
| --- | --- | --- | --- | --- | --- | --- |
| 200 | 1.7922 | 0.1372 | 4.5 | 0.80 | 18.3688 | 159.1844 |
| 180 | 2.5777 | 0.1973 | 6 | 0.77 | 36.5447 | 168.2724 |
| 173 | 2.9652 | 0.2270 | 6.5 | 0.74 | 47.2402 | 173.6201 |

Figure 12-21: maximum-film-pressure ratio

ppmax=0.3

pmax=555.5556 psi

The peak film pressure:

ptotal=pmax+ps=555.5556 psi+60 psi=615.5556 psi

The minimum film thickness:

h0=1-ϵc=5.98×10-4 in

The lubricant side-flow rate:

Qs=πpsrc33μl'1+1.5ϵ2=0.5870 in3/s

The heat loss rate:

Hloss=ρCpQs∆T=0.3622 Btu/s=1303.9 Btu/h

| c (in) | Tav (℉) | ptotal (psi) | h0(10-4) (in) | Hloss (Btu/h) | Qs (in3/s) |
| --- | --- | --- | --- | --- | --- |
| 0.0015 | 198 | 580.8333 | 5.40 | 1046.6 | 0.2308 |
| 0.0023 | 173 | 615.5556 | 5.98 | 1303.9 | 0.5870 |

Comparison: The greater the clearance, the smaller the film temperature, the greater the peak film pressure, the greater the minimum film thickness, the greater the lubricant side-flow rate, and the greater the heat loss rate.

- [minimum clearance]

| Trial Tf | μ (10-6) | S | frc | ϵ | ∆Tf | Tav |
| --- | --- | --- | --- | --- | --- | --- |
| 198 | 1.5961 | 0.3338 | 8 | 0.64 | 96.4480 | 198.2240 |

Qs=0.2308 in3/s

Figure 12-21: flow ratio

QsQ=0.80

The required capacity of the oil pump:

Q=0.2885 in3/s

[maximum clearance]

| Trial Tf | μ (10-6) | S | frc | ϵ | ∆Tf | Tav |
| --- | --- | --- | --- | --- | --- | --- |
| 173 | 2.9652 | 0.2270 | 6.5 | 0.74 | 47.2402 | 173.6201 |

Qs=0.5870 in3/s

Figure 12-21: flow ratio

QsQ=0.84

The required capacity of the oil pump:

Q=0.6988 in3/s

- [minimum clearance]

Hloss=1046.6 Btu/h

[maximum clearance]

Hloss=1303.9 Btu/h

- [minimum clearance]

h0=5.40×10-4 in>0.002+0.00004⋅d=3.200×10-4 in

Tmax=Ts+∆T=150℉+96.4480℉=246.4480℉<250℉

[maximum clearance]

h0=5.98×10-4 in>0.002+0.00004⋅d=3.200×10-4 in

Tmax=Ts+∆T=150℉+47.2402℉=97.2402℉<250℉

Therefore, the bearing will survive the operation.