Christopher King

2018141521058

Mechanical Design 2

Class Section 01

11/17/2021

# Problem 1

A single-threaded 25-mm power screw is 25 mm in diameter with a pitch of 5 mm. A vertical load on the screw reaches a maximum of 5 kN. The coefficients of friction are 0.06 for the collar and 0.09 for the threads. The frictional diameter of the collar is 45 mm.

- Find the torque to “raise” and “lower” the load.

- Find the overall efficiency.

Solution:

dm=d-p2=22.5 mm

Single-threaded: l=p=5 mm

TR=Fdm2πfdm+lπdm-fl+Ffcdc2=5 kN×22.5 mm2×π×0.09×22.5 mm+5 mmπ×22.5 mm-0.09×5 mm+5 kN×0.06×45 mm2=15.8493 N⋅m

TL=Fdm2πfdm-lπdm+fl+Ffcdc2=5 kN×22.5 mm2×π×0.09×22.5 mm-5 mmπ×22.5 mm+0.09×5 mm+5 kN×0.06×45 mm2=7.8268 N⋅m

e=T0TR=Fl2πTR=5 kN×5 mm2π×15.8493 N⋅m=0.2510

# Problem 2

The press shown in figure below has a rated load of 5000 lbf. The twin screws have Acme threads, a diameter of 2 in, and a pitch of 1/4 in. Coefficients of friction are 0.05 for the threads and 0.08 for the collar bearings. Collar diameters are 3.5 in. The gears have an efficiency of 95 percent and a speed ratio of 60:1. A slip clutch, on the motor shaft, prevents overloading. The full-load motor speed is 1720 rev/min.

- When the motor is turned on, how fast will the press head move?

- What should be the horsepower rating of the motor?

Solution:

Single-threaded: l=p=0.25 in

n=1720 rpm60

v=nl=1720 rpm60×0.25 in=7.1667 in/min

dm=d-p2=2-0.252=1.875 in

TR=Fdm2πfdmsecα+lπdm-flsecα+Ffcdc2=2500 lbf×1.875 in2×π×0.05×1.875 in×sec29°2+0.25 inπ×1.875 in-0.05×0.25 in×sec29°2+2500 lbf×0.08×3.5 in2=570.9992 lbf⋅in

Tmotor=2TR60×0.95=20.0351 lbf⋅in

H=Tmotorn63025=20.0351 lbf⋅in×1720 rpm63025=0.5468 hp

# Problem 3

A double square-thread power screw has an input power of 3 kW at a speed of 1 rev/s. The screw has a diameter of 40 mm and a pitch of 8 mm. The frictional coefficients are 0.14 for the threads and 0.09 for the collar, with a collar friction radius of 50 mm. Determine

- the axial resisting load F and the combined efficiency of the screw and collar;

- axial stress and torsional stress on thread body;

With calculated F, assume the first-engaged thread takes 40 percent of the axial load, calculate the followings:

- bearing stress on thread flank;

- max shear stress across puncture plane of the thread;

- thread bending stress at the root of the thread;

- von Mises stress at the root of the thread; and

- draw a 3D stress element to show the direction of all component stresses acting on element at the root.

Solution:

Double-threaded: l=2p=16 mm

dm=d-p2=36 mm

TR=Fdm2πfdm+lπdm-fl+Ffcdc2=F×36 mm2×π×0.14×36 mm+16 mmπ×36 mm-0.14×16 mm+F×0.09×100 mm2=0.0097F

ω=2πn=2π rad/s

H=Tω=2π×0.0097F=3 kW

⇒T=Hω=3 kW2π rad/s=477.4648 N⋅m

⇒F=49.382 kN

e=T0TR=Fl2πTR=49.382 kN×16 mm2π×477.4648 N⋅m=0.2634

dr=d-p=32 mm

Axial stress:

σ=FAt=-49.382 kNπ×32 mm24=-61.401 MPa

Torsional stress:

τ=16Tπdr3=16×477.4648 N⋅mπ×32 mm3=74.210 MPa

Bearing stress:

σB=-40%Fπdmp2=-40%×49.382 kNπ×36 mm×8 mm2=-43.663 MPa

Max shear stress

τ=3⋅40%Fπdmp=-3×40%×49.382 kNπ×36 mm×8 mm=65.495 MPa

Thread bending stress:

σb=McI=6⋅40%Fπdrp=147.36 MPa

von Mises stress:

σx=147.36 MPa

σy=-61.401 MPa

σz=0 MPa

τxy=0 MPa

τyz=74.210 MPa

τzx=-4⋅40%Tπdr2p=-29.684 MPa

σe=12σx-σy2+σy-σz2+σz-σx2+6τxy2+τyz2+τzx212=231.73 MPa

z

σx

τzx

τyz

σy

y

x