Christopher King

2018141521058

Mechanical Design 2

Class Section 01

11/28/2021

# Problem 1

Draw the internal and external thread profiles of a ¼ -20 – UNC thread and identify the key dimensions.

Solution:

Nominal major diameter of 0.25 in and 20 threads per inch.

Tensile Stress Area: 0.0318 in2.

p=120 /inch=0.05 inch

dr=d-1.299038p=0.1850481

dp=d-0.649519p=0.21752405

# Problem 2

An M14X2 hex-head bolt with a nut is used to clamp together two 15-mm steel plates.

- Select a proper class R washer for the bolt and nut.

- Determine a suitable length of the bolt, rounded up to the nearest 5 mm.

- Determine the bolt stiffness.

- Determine the stiffness of the member per Eq. 8-19.

- Repeat (c) using Eq. 8-22 and Eq. 8-23.

Solution:

Nominal major diameter of 14 mm and pitch of 2 mm.

dp=14 mm

Therefore, I select washer size 14 R from Table A-33 with maximum thickness 3.50 mm.

From Table A-31, the nut height is H=12.8 mm.

Hence,

l=2t+2hw=2×15 mm+3.50 mm=33.5 mm

L≥l+H=2×15 mm+12.8 mm+3.50 mm=46.3000 mm

Rounded up to the nearest 5 mm,

L=50 mm

The thread length of inch-series bolts is

LT=2d+6=2×14 mm+6 mm=34 mm

Length of unthreaded portion in grip:

ld=L-LT=50 mm-34 mm=16 mm

Length of threaded portion in grip:

lt=l-ld=33.5 mm-16 mm=17.5 mm

Area of unthreaded portion:

Ad=πd24=π×14 mm24=153.9380 mm2

Area of threaded portion (from Table 8-1):

At=115 mm2

Fastener stiffness:

kb=AdAtEAdlt+Atld=153.9380 mm2×115 mm2×207 GPa153.9380 mm2×14 mm+115 mm2×16 mm=9.1723×108 N/m

Eq. 8-19: the spring rate or stiffness of upper frustum

k1=Pδ=πEdtanαln2ttanα+D-dD+d2ttanα+D+dD-d=π×207 GPa×14 mm×tan30°ln2×15 mm×tan30°+21 mm-14 mm21 mm+14 mm2×15 mm×tan30°+21 mm+14 mm21 mm-14 mm=6.2326×109 N/m

Eq. 8-19: the spring rate or stiffness of middle frustum

D=21 mm+2×15 mm×tan30°=38.32 mm

k2=Pδ=πEdtanαln2ttanα+D-dD+d2ttanα+D+dD-d=π×207 GPa×14 mm×tan30°ln2×1.75 mm×tan30°+38.32 mm-14 mm38.32 mm+14 mm2×1.75 mm×tan30°+38.32 mm+14 mm38.32 mm-14 mm=1.2539×1011 N/m

Eq. 8-19: the spring rate or stiffness of lower frustum

k3=Pδ=πEdtanαln2ttanα+D-dD+d2ttanα+D+dD-d=π×207 GPa×14 mm×tan30°ln2×16.75 mm×tan30°+21 mm-14 mm21 mm+14 mm2×16.75 mm×tan30°+21 mm+14 mm21 mm-14 mm=5.9375×109 N/m

Total member stiffness:

km=1k1+1k2+1k3-1=2.9687×109 N/m

Eq. 8-22:

km=0.5774πEd2ln50.5774l+0.5d0.5774l+2.5d=0.5774π×207 GPa×14 mm2ln50.5774×33.5 mm+0.5×14 mm0.5774×33.5 mm+2.5×14 mm=2.9689×109 N/m

Eq. 8-23:

km=EdAexpBdl=207 GPa×14 mm×0.78715×exp0.62873×14 mm33.5 mm=2.9667×109 N/m

(Table 8-8 (Steel): A=0.78715, B=0.62873)

# Problem 3

A 30-mm thick AISI 1020 steel plate is sandwiched between a 10-mm thick 2024-T3 aluminum plate on the top and another 20-mm thick 2024-T3 aluminum plate on the bottom. The plates are tightened with a bolt and nut as well as washers of class N under both. The bolt is M10 × 1.5, property class 5.8.

- Assuming the bolt requires its length to be at least 2 threads beyond the nut after tightening. Determine the minimum length of the bolts per Table A-17.

- Determine the bolt stiffness.

- Determine the stiffness of the members using Eq. 8-20.

- Determine the stiffness of the joint.

Solution:

Nominal diameter of bolt is 10 mm.

Therefore, I select washer size 14 N from Table A-33 with maximum thickness 2.30 mm.

From Table A-31, the nut height is H=8.4 mm.

Hence,

l=l1+l2+l3+hw=30 mm+10 mm+20 mm+2.30 mm=62.3 mm

L≥l+H=62.3 mm+8.4 mm=70.7 mm

The minimum length of the bolts per Table A-17 is equal to

L=80 mm

The thread length of inch-series bolts is (Table 8-7)

LT=2d+6=2×10 mm+6 mm=26 mm

Length of unthreaded portion in grip:

ld=L-LT=80 mm-26 mm=54 mm

Length of threaded portion in grip:

lt=l-ld=62.3 mm-54 mm=8.3 mm

Area of unthreaded portion:

Ad=πd24=π×10 mm24=78.540 mm2

Area of threaded portion (from Table 8-1):

At=58.0 mm2

Fastener stiffness:

kb=AdAtEAdlt+Atld=78.540 mm2×58.0 mm2×207 GPa78.540 mm2×8.3 mm+58.0 mm2×54 mm=2.4920×108 N/m

Eq. 8-20: the spring rate or stiffness of upper aluminum frustum

k1=Pδ=πEdtanαln2ttanα+D-dD+d2ttanα+D+dD-d=π×71 GPa×10 mm×tan30°ln2×12.30 mm×tan30°+15 mm-10 mm15 mm+10 mm2×12.30 mm×tan30°+15 mm+10 mm15 mm-10 mm=1.4377×109 N/m

Eq. 8-20: the spring rate or stiffness of upper steel frustum

D=15 mm+2×12.3 mm×tan30°=29.2 mm

k2=Pδ=πEdtanαln2ttanα+D-dD+d2ttanα+D+dD-d=π×207 GPa×10 mm×tan30°ln2×18.8 mm×tan30°+29.2 mm-10 mm29.2 mm+10 mm2×18.8 mm×tan30°+29.2 mm+10 mm29.2 mm-10 mm=1.1876×1010 N/m

Eq. 8-20: the spring rate or stiffness of lower steel frustum

D=15 mm+2×20 mm×tan30°=29.2 mm

k3=Pδ=πEdtanαln2ttanα+D-dD+d2ttanα+D+dD-d=π×207 GPa×10 mm×tan30°ln2×11.1 mm×tan30°+21 mm-10 mm21 mm+10 mm2×11.1 mm×tan30°+21 mm+10 mm21 mm-10 mm=2.6809×1010 N/m

Eq. 8-20: the spring rate or stiffness of upper aluminum frustum

k4=Pδ=πEdtanαln2ttanα+D-dD+d2ttanα+D+dD-d=π×71 GPa×10 mm×tan30°ln2×20 mm×tan30°+15 mm-10 mm15 mm+10 mm2×20 mm×tan30°+15 mm+10 mm15 mm-10 mm=1.2015×109 N/m

Total member stiffness:

km=1k1+1k2+1k3+1k4-1=6.0629×108 N/m

The joint stiffness is equal to

kj=kb+km=2.4920×108 N/m+6.0629×108 N/m=8.5549×108 N/m

# Problem 4

Assuming the bolt requires its length to be at least 2 threads beyond its grip length. Repeat Prob. #3 (a) thru (d) with the bottom aluminum plate having a threaded hole to eliminate the nut.

Solution:

Nominal diameter of bolt is 10 mm.

Therefore, I select washer size 14 N from Table A-33 with maximum thickness 2.30 mm.

Hence,

l=l1+l2+d2+hw=30 mm+10 mm+10 mm2+2.30 mm=47.3 mm

L≥h+1.5d=30 mm+10 mm+2.30 mm+1.5×10 mm=57.3 mm

The minimum length of the bolts per Table A-17 is equal to

L=60 mm

The thread length of inch-series bolts is (Table 8-7)

LT=2d+6=2×10 mm+6 mm=26 mm

Length of unthreaded portion in grip:

ld=L-LT=60 mm-26 mm=34 mm

Length of threaded portion in grip:

lt=l-ld=47.3 mm-34 mm=13.3 mm

Area of unthreaded portion:

Ad=πd24=π×10 mm24=78.540 mm2

Area of threaded portion (from Table 8-1):

At=58.0 mm2

Fastener stiffness:

kb=AdAtEAdlt+Atld=78.540 mm2×58.0 mm2×207 GPa78.540 mm2×13.3 mm+58.0 mm2×34 mm=3.1259×108 N/m

Eq. 8-20: the spring rate or stiffness of upper aluminum frustum

k1=Pδ=πEdtanαln2ttanα+D-dD+d2ttanα+D+dD-d=π×71 GPa×10 mm×tan30°ln2×12.30 mm×tan30°+15 mm-10 mm15 mm+10 mm2×12.30 mm×tan30°+15 mm+10 mm15 mm-10 mm=1.4377×109 N/m

Eq. 8-20: the spring rate or stiffness of upper steel frustum

D=15 mm+2×12.3 mm×tan30°=29.2 mm

k2=Pδ=πEdtanαln2ttanα+D-dD+d2ttanα+D+dD-d=π×207 GPa×10 mm×tan30°ln2×17.7 mm×tan30°+29.2 mm-10 mm29.2 mm+10 mm2×17.7 mm×tan30°+29.2 mm+10 mm29.2 mm-10 mm=1.2301×1010 N/m

Eq. 8-20: the spring rate or stiffness of lower steel frustum

D=15 mm+2×60 mm-30 mm-10 mm-2.3 mm×tan30°=35.4 mm

k3=Pδ=πEdtanαln2ttanα+D-dD+d2ttanα+D+dD-d=π×207 GPa×14 mm×tan30°ln2×12.3 mm×tan30°+35.4 mm-14 mm35.4 mm+14 mm2×12.3 mm×tan30°+35.4 mm+14 mm35.4 mm-14 mm=2.1877×1010 N/m

Eq. 8-20: the spring rate or stiffness of upper aluminum frustum

k4=Pδ=πEdtanαln2ttanα+D-dD+d2ttanα+D+dD-d=π×71 GPa×10 mm×tan30°ln2×17.7 mm×tan30°+15 mm-10 mm15 mm+10 mm2×17.7 mm×tan30°+15 mm+10 mm15 mm-10 mm=1.2511×109 N/m

Total member stiffness:

km=1k1+1k2+1k3+1k4-1=6.1657×108 N/m

The joint stiffness is equal to

kj=kb+km=2.4920×108 N/m+6.0629×108 N/m=9.2916×108 N/m