Christopher King

2018141521058

Mechanical Design 2

Class Section 01

12/03/2021

# Problem 1

For a bolted assembly with six bolts, the stiffness of each bolt is kb = 3 Mlbf/in and the stiffness of the members is km = 12 Mlbf/in per bolt. An external load of 80 kips is applied to the entire joint. Assume the load is equally distributed to all the bolts. It has been determined to use 1/2 in-13 UNC SAE grade 8 bolts with rolled threads. Assume the bolts are preloaded to 75 percent of the proof load.

It is desired to find the range of torque that could apply to initially preload the bolts without expecting failure once the joint is loaded. Assume a torque coefficient of K = 0.2.

- Determine the maximum bolt preload that can be applied without exceeding the proof strength of the bolts.

- Determine the minimum bolt preload that can be applied while avoiding joint separation.

- Determine the value of torque in units of ft-lbf that should be specified for preloading the bolts if it is desired to preload to the midpoint of the values found in parts (a) and (b).

Solution:

1/2 in-13 UNC SAE grade 8 bolts with rolled threads. Assume a torque coefficient of K = 0.2.

Proof strength (From Table 8-9):

Sp=120 kpsi

Tensile stress area (From Table 8-2):

At=0.1419 in2

The maximum bolt preload is equal to

Fi=SpAt=120 kpsi×0.1419 in2=17.028 kips

The stiffness constant of the joint:

C=kbkb+km=3 Mlbf/in3 Mlbf/in+12 Mlbf/in=0.2

The load P is

P=PtotalN=80 kips6=13.3 kips

Joint separation, Eq. (8-30) with n0=1, we can know the minimum bolt preload is equal to

Fi=n0P1-C=1×13.3 kips×1-0.2=10.667 kips

The midpoint of the values found in parts (a) and (b) is equal to

Fi=17.028 kips+10.667 kips2=13.847 kips

The value of torque in units of ft-lbf that should be specified for preloading the bolts is equal to

T=KFid=0.2×13.847 kips×0.5 in=115.3944 lbf⋅ft

# Problem 2

A gas storage tank shown in figure below has a steel end cap bolt-tightened to a grade 30 cast iron flange. The end cap has to be removed frequently for maintenance purpose. A non-permanent connection with eight 7/16 in-14 UNC SAE grade 8 bolts was selected for the purpose. Gas pressure inside the tank results in a maximum force up to 10 kips on the end cap.

For the given bolt specifications, with two threads beyond the nut, select a suitable bolt length from the preferred sizes in Table A–17. Determine the yielding factor of safety np, the load factor nL, and the joint separation factor n0.

Solution:

Nominal major diameter of 716 in.

Therefore, I select washer size 716 W from Table A-33 with maximum thickness 0.083 inch.

From Table A-31, the nut height is H=38 in.

Because for the given bolt specifications, with two threads beyond the nut,

l=t1+t2+2hw=38 in+12 in+2×0.083 inch=1.0410 inch

L≥l+H+2p=1.0410 inch+38 in+2×114 in=1.5589 inch

The minimum length of the bolts per Table A-17 is equal to

L=1.60 inch

The thread length of inch-series bolts is (Table 8-7)

LT=2d+14=2×716 in+14 in=1.1250 in

Length of unthreaded portion in grip:

ld=L-LT=1.60 in-1.1250 in=0.4750 in

Length of threaded portion in grip:

lt=l-ld=1.0410 inch-0.4750 in=0.5660 in

Area of unthreaded portion:

Ad=πd24=π×716 in24=0.1503 in2

Area of threaded portion (from Table 8-1):

At=0.1063 in2

Fastener stiffness:

kb=AdAtEAdlt+Atld=0.1503 in2×0.1063 in2×30 Mpsi0.1503 in2×0.5660 in+0.1063 in2×0.4750 in=3.5360×106 lbf/in

Eq. 8-20: the spring rate or stiffness of steel frustum

k1=Pδ=πEdtanαln2ttanα+D-dD+d2ttanα+D+dD-d=π×30 Mpsi×716 in×tan30°ln2×0.4400 in×tan30°+0.6563 in-716 in0.6563 in+716 in2×0.4400 in×tan30°+0.6563 in+716 in0.6563 in-716 in=2.8527×107 lbf/in

Eq. 8-20: the spring rate or stiffness of upper cast iron frustum

D=0.6563 in+2×0.4400 in×tan30°=1.1851 in

k2=Pδ=πEdtanαln2ttanα+D-dD+d2ttanα+D+dD-d=π×14.5 Mpsi×716 in×tan30°ln2×0.0625 in×tan30°+1.1851 in-716 in1.1851 in+716 in2×0.0625 in×tan30°+1.1851 in+716 in1.1851 in-716 in=2.3657×108 lbf/in

Eq. 8-20: the spring rate or stiffness of lower cast iron frustum

k3=Pδ=πEdtanαln2ttanα+D-dD+d2ttanα+D+dD-d=π×14.5 Mpsi×716 in×tan30°ln2×0.4400 in×tan30°+0.6563 in-716 in0.6563 in+716 in2×0.4400 in×tan30°+0.6563 in+716 in0.6563 in-716 in=1.3029×107 lbf/in

Total member stiffness:

km=1k1+1k2+1k3-1=8.6180×106 lbf/in

The stiffness constant of the joint:

C=kbkb+km=3.5360×106 lbf/in3.5360×106 lbf/in+8.6180×106 lbf/in=0.2909

Proof strength (From Table 8-9):

Sp=120 kpsi

Because the end cap has to be removed frequently for maintenance purpose, it is non-permanent connection.

From Eq. 8-31 and 8-32, I can know that

Fi=0.75Fp=0.75AtSp=0.75×0.1063 in2×120 kpsi=9.567 kips

The load P is

P=PtotalN=10 kips8=1.250 kips

Yielding factor of safety, Eq. 8-28:

np=SpAtCP+Fi=120 kpsi×0.1063 in20.2909×1.250 kips+9.567 kips=1.2845

Overload factor of safety, Eq. 8-29:

nL=SpAt-FiCP=120 kpsi×0.1063 in2-9.567 kips0.2909×1.250 kips=8.7691

Separation factor of safety, Eq. 8-30:

n0=FiP1-C=9.567 kips1.250 kips×1-0.2909=10.7939

# Problem 3

Continued from Question 02, the gas force on end cap is cycled between 0 and 10 kips. Determine the fatigue factor of safety for the bolts using failure criteria of (a) Goodman, and (b) Gerber.

Among all considered failure scenarios, which mode is the riskiest for failure?

Solution:

σi=0.75Sp=90 kpsi

σa=CPmax-Pmin2At=CP2At=0.2909×1.250 kips2×0.1063 in2=1.7106 kpsi

σm=CPmax+Pmin2At+σi=CP2At+σi=0.2909×1.250 kips2×0.1063 in2+σi=91.711 kpsi

Goodman: From Table 8-9, Sut=150 kpsi, and from Table 8-17, Se=23.2 kpsi

Eq. 8-45:

nf=SeSut-σiσaSut+Se=23.2 kpsi×150 kpsi-90 kpsi1.7106 kpsi×150 kpsi+23.2 kpsi=4.6984

Gerber:

Eq. 8-46:

nf=12σaSeSutSut2+4SeSe+σi-Sut2-2σiSe=12×1.7106 kpsi×23.2 kpsi×150 kpsi×150 kpsi2+4×23.2 kpsi×23.2 kpsi+90 kpsi-150 kpsi2-2×90 kpsi×23.2 kpsi=7.2433

Yielding is the riskiest for failure.