Christopher King

2018141521058

Mechanical Design 2

Class Section 01

12/19/2021

# Problem 1

Given: The teeth on a pair of mating gears have a 6 diametral pitch, 20° pressure angle AGMA involute form. The pinion has 19 teeth, and the gear has 37 teeth. Find the following parameters:

- Gear ratio

- Circular pitch

- Base pitch

- Pitch diameter of pinion and gear

- Center distance

- Addendum

- Dedendum

- Whole tooth depth

- Clearance

- Outside diameter of pinion and gear

- Base diameter of pinion and gear

- Contact ratio

Solution:

P=6 teeth/inch

ϕ=20°

Np=19 teeth

Ng=37 teeth

- Gear ratio: NgNp=3719

- Circular pitch: p=πP=π6 in

- Base pitch: pb=pcosϕ=π6×cos20°=0.4920

- Pitch diameter of pinion and gear: dp=NpP=196, dg=NgP=376

- Center distance: dp+dg2=143

- Addendum: a=1P=16

- Dedendum: b=1.25P=524

- Whole tooth depth: ht=ab=38

- Clearance: c=b-a=124

- Outside diameter of pinion and gear: dop=dp+2a=72, dog=dg+2a=132

- Base diameter of pinion and gear: dbp=dpcosϕ=196×cos20°=2.9757, dbp=dopcosϕ=376×cos20°=5.7948

- Contact ratio: 
mc=rap2-rbp2+rag2-rbg2-Csinϕpb=742-2.975722+1342-5.794822-143sin20°0.4920=1.6209

# Problem 2

For a pair of spur gears with gear ratio of 4:1, specify the minimum number of teeth allowed on the pinion to avoid the problem of interference assuming full-depth tooth depth using

- a 20° pressure angle, and

- a 25° pressure angle.

Solution:

Using Eq. 13-11 with k=1, ϕ=20°, and m=4,

Np=2k1+2msin2ϕm+m2+1+2msin2ϕ=15.44 teeth

Rounding up,

Np=16 teeth

Using Eq. 13-11 with k=1, ϕ=25°, and m=4,

Np=2k1+2msin2ϕm+m2+1+2msin2ϕ=10.20 teeth

Rounding up,

Np=11 teeth

# Problem 3

Shaft a in the figure has a power input of 75 kW at a speed of 1000 rev/min in the counterclockwise direction. The gears have a module of 5 mm and a 20° pressure angle. Gear 3 is an idler.

- Find the force F3b that gear 3 exerts against shaft b.

- Find the torque T4c that gear 4 exerts on shaft c.

Solution:

ω=2πn60

H=Tω=2πTn60

Ta=60H2πn=716.220334 N⋅m

r2=mN22=42.5 mm

F32t=Tar2=16.85 kN

F3b=-Fb3=2×16.85=33.70 kN

which is in the positive x-direction.

r4=mN42=127.5 mm

Tc4=F32tr4=2148.59 N⋅m ccw

T4c=2148.59 N⋅m cw

# Problem 4

For the countershaft shown below, Gear A receives power from another gear with the transmitted force FA applied at the 20° pressure angle as shown. The power is transmitted through the shaft and delivered through gear B through a transmitted force FB at the pressure angle shown.

Assume the gear ratio from gear B to its mating gear is 2 to 1.

- Determine the minimum number of teeth that can be used on gear B without an interference problem in the teeth.

- Using the number of teeth from part (a), what diametral pitch is required to also achieve the given 8-in pitch diameter?

- Suppose the 20° pressure angle gears are exchanged for gears with 25° pressure angle, while maintaining the same pitch diameters and diametral pitch. Determine the new forces FA and FB if the same power is to be transmitted.

Solution:

Using Eq. 13-11 with k=1, ϕ=20°, and m=2,

Np=2k1+2msin2ϕm+m2+1+2msin2ϕ=14.16 teeth

Rounding up,

Np=15 teeth

P=Nd=158=1.875 teeth/in

With ϕ=20°

WtA=FAcos20°=281.9 lbf

With ϕ=25°

FA'=WtAcos25°=311.0 lbf

And

WtAdA2=WtBdB2

WtB=704.75 lbf

FB'=WtBcos25°=777.6 lbf